# AUTOTAF - AUTOMATIC TAF PRODUCTION

## Diederik Haalman

METEO SERVICE weather research GmbH

Teltower Damm 25, 14169 Berlin, Germany

Phone +49 30 8129 9015,    Fax:+49 30 8129 9016, email D.Haalman@metserv.de

## Autotaf

The Autotaf is a TAF produced by a code generator based on MOS forecasts (Knüpffer 1997) without any human intervention. Autotaf was developed in cooperation with the DWD (Deutscher Wetterdienst) and the KNMI (Royal Netherlands Meteorological Institute) who are using it as a guidance for the forecaster.

Four main targets were set for Autotaf. These are in order of importance:

- syntax and semantics complying with ICAO regulations (ICAO 1998)

- internal meteorological consistency

- a style indistinguishable from a human made TAF

- a verified forecast quality about equal to the issued TAF or even better.

## TAF

Aviation forecasts are an important part of applied meteorology. The TAF code is an internationally accepted standard for aerodrome forecasts. It contains probabilistic information on more than fifty categories divided over visibility, ceiling, wind and the type of weather. About twenty-five categories are significant for air traffic and have direct consequences    for the starting rights of aircraft with the aerodrome as destination. An over forecast may result in significant economic damage due to cancelled flights. An under forecast may result in even more damage due to many aircraft waiting in mid air for permission to land on an aerodrome with reduced landing capacity.

## Autotaf history

In 1995 Meteoservice developed a MOS system in cooperation with the DWD making automated forecasts for most of the TAF categories. In 1996 a simple TAF code generator was added presenting the MOS forecast in TAF syntax. It soon turned out that there are many more requirements for a TAF than just the syntax and that the properties of the MOS do not automatically fulfil these. End 1997 a new prototype TAF code generator was build. It was a rule based program. Although being far from perfect, it proved that it is possible to produce an automatic TAF with a human look and feel and a high verified quality. In 1998 the UKMO    used the program to generate TAFs based on a high resolution local area model instead of MOS. They performed extended tests revealing the limits of the prototype. Also in 1998 the KNMI    stepped into the project. They implemented the MOS System and several extension were developed. In December 2000 the prototype TAF code generator was replaced by the “mtaf“ program which complies with an extended set of requirements.

## Syntax and semantics

A subset of the syntax is implemented containing all common praxis elements.

Used elements:

surface wind in knots, horizontal visibility, clouds

weather: NSW, BL, SH, TS, FZ, DZ, RA, SN, GR, GS, BR, FG, HZ, +, -

change groups: BECMG, TEMPO, PROB 40, PROB30

Omitted elements:

KMH    MPS , vertical visibility, temperature, icing, turbulence

weather: VC, MI, BC, DR, SG, IC, PE, FU, VA, DU, SA, PO, SQ, FC, SS, DS

change groups: FM

The semantics were the most difficult part of creating a TAF code generator. The ICAO specifications are not clear in this aspect. The probabilistic nature of TAFs and TAF-MOS makes them sometimes difficult to understand and there is no consensus among forecasters. I will discuss one aspect in detail.

The ICAO rule states that the most likely value is reported in the base or behind BECMG. But what is the most likely value? In the Autotaf we use the value that has equal chances to be higher or lower than the actual value, referred to as the 50% reliable value. Note that this does not guarantee a bias free forecast for each category. What happens if we add a PROB30 group?    Should we use the remaining 70% for the base? No, the Autotaf program still uses 50% because the most likely value does not change when a less likely lower value is reported in the TAF. Please note that using 65% might give the best verification results since it is halfway between 30% and 100%.

Several aerodrome dependant aspects like the highest minimum sector altitude, significant categories and coding rules are handled by the mtaf program in its configuration.

Meteorological consistency and human style

Style is a very subjective aspect but was implemented to satisfaction. In general less likely events are omitted in favour of more likely events. However during a rapid change of conditions the mtaf program may not find the best solution.

The meteorological consistency poses another problem. Some details needed for PROB or TEMPO groups were missing from the MOS. A human being makes up for the missing information with his experience. In the code generator this was solved by adding expert information on visibility and ceiling during precipitation.

## Verified forecast quality

The forecast quality of the Autotaf largely depends on the quality of the MOS input to the code generator. The KNMI made verification result available based on data for Amsterdam airport (EHAM) from the year 1999. Visibility and ceiling are verified using RPS (Ranked Probability Score) and Wind is verified using PWE (Probability Weighted Error). Four forecasting methods are compared:

- TAF	The TAF as issued by the forecaster

- MOSP	MOS with exact probabilities for each category

- MOS1 	MOS using only a single value

- PERS	Persistency of the latest observation as was available to the MOS

These values are not 100% comparable since they have different ways to express uncertainty. MOS1 does not use uncertainty at all. TAF can use PROB, TEMPO and BECMG. MOSP can use any probability between 0% and 100%. Autotaf is expected to have a quality somewhere in-between MOSP and MOS1.

In the first hour persistency is better than MOS. This is caused by the fact that MOS    was issued one hour earlier giving both the persistency and the TAF a one hour advantage. During the first 4 hours TAF is better on all elements. Somewhere near the 5th hour MOSP    starts being better than TAF. Around the 9th hour    MOS1 becomes equally good as TAF and for some elements even better. Only for strong winds this effect does not show.

A recent verification by the DWD of the Autotafs produced by the mtaf program showed promising results in almost all aspects. More verification over a longer period is needed to confirm these results.

## Example

TAF-MOS   Fr 06.04.2001 calculated at 08:20 UTC for  EDDH Hamburg

----------------------------------------------------------------------

! UTC TIME                      !

PREDICTAND                      UNIT ! 10 11 12 13 14 15 16 17 18 19 !

-------------------------------------!-------------------------------!

wind speed                     knots ! 10 11 12 13 12 12 11 10  9  9 !

direction                  10 degree ! 22 23 24 24 24 24 24 23 23 21 !

-------------------------------------!-------------------------------!

visibility                      100m ! 51 56 66 77 OK OK OK OK OK OK !

visibility during precipitation 100m ! 34 41 46 56 71 82 85 89 90 82 !

!                               !

probability of visibility < 5000m  % ! 49 46 40 32 27 26 21 20 16 15 !

probability of visibility < 3000m  % ! 31 27 22 16 13 12 10 11  6  6 !

probability of visibility < 1500m  % ! 12  6  6  5  3  4  3  3  1  1 !

probability of visibility <  800m  % !  0  0  0  0  0  0  0  0  0  0 !

-------------------------------------!-------------------------------!

Overcast                       100ft ! OK OK OK OK OK OK OK OK OK OK !

Broken                         100ft !  8  8  9 11 13 19 29 34 41 39 !

Scattered                      100ft !  7  8  8  9 11 16 22 24 33 37 !

Ceiling during precipitation   100ft !  7  8  9 11 13 15 15 15 15 17 !

!                               !

probability of Ceiling < 5000ft    % ! 88 84 84 81 79 77 71 63 58 62 !

probability of Ceiling < 1500ft    % ! 86 79 73 68 56 46 36 33 24 21 !

probability of Ceiling < 1000ft    % ! 73 65 57 43 27 16 19 21 24 21 !

probability of Ceiling <  500ft    % ! 14 11  3  0  0  0  0  0  0  1 !

probability of Ceiling <  200ft    % !  0  0  0  0  0  0  0  0  0  0 !

-------------------------------------!-------------------------------!

probability of precipitation       % ! 79 70 57 52 41 35 26 19 17 18 !

probability of moderate or heavy   % !  9 10 11  7  7  4  2  1  1  0 !

probability of TEMPO precipitation % ! 83 81 83 82 71 62 51 42 40 38 !

probability of stratiform          % ! 75 69 63 61 48 36 28 23 17 20 !

probability of convective          % !  8 12 19 21 24 26 23 19 23 18 !

probability of thunderstorm        % !  0  0  0  0  0  0  0  0  0  0 !

probability of solid               % !  0  0  1  0  0  0  0  0  0  0 !

probability of liquid              % ! 83 81 81 81 71 61 51 42 40 38 !

-------------------------------------!-------------------------------!

The Autotaf generated for Hamburg on April 6 is used as an example. For reasons of simplification all irrelevant predictands are left out of the example MOS. The first step is to identify all significant changes .

BKN009 BECMG 1213 BKN011

BKN013 BECMG 1415 BKN019

These can be merged into one BECMG group.

BKN009 BECMG 1215 BKN019

Than the base with optimised values is calculated:

EDDH 060900Z 061019 23011KT 9000 BKN008

BECMG 1215 SCT020 BKN030=

A visibility between 3400 and 4600 during light precipitation in the first two hours is forecasted. Light precipitation itself is not significant but the visibility going with it is. A TEMPO group is added.

TEMPO 1012 4000 -RA

Several PROB groups are proposed.

PROB30 1010 2000 BR

PROB40 1517 BKN014

None of these are important enough to be added to the Autotaf.    The final Autotaf is:

EDDH 060900Z 061019 23011KT 9000 BKN008

TEMPO 1012 4000 -RA

BECMG 1215 SCT020 BKN030=

The forecaster TAF was:

EDDH 060900Z 061019 23010KT 4000 BKN007

TEMPO 1012 2500 RADZ

BKN003 BECMG 1215 8000 BKN015=

## Conclusion

All four targets have been achieved. Still further improvements are expected. Autotaf can be considered as an alternative to the human made TAF but more verification is required to validate recent improvements.

Automatic TAF generation seems to have commercial potential since many weather services have shown interest. However, an unsupervised automatic TAF is not to be expected any soon because of the huge responsibility connected to the TAF. The weather services intent to use the Autotaf as a guidance for the forecaster, freeing him in most cases from examining the larger MOS output.

## Acknowledgement

We thank the DWD and KNMI for their role in the Autotaf development. Special thanks to dr. Nico Maat from the KNMI for the many discussions on TAF semantics. Thanks to all who made the Autotaf project possible.

REFERENCES

ICAO, 1998 :”METEOROLOGICAL SERVICE FOR INTERNATIONAL AIR NAVIGATION – ANNEX 3 TO THE CONVENTION ON INTERNATIONAL CIVIL AVIATION, THIRTEENTH EDITION –JULY 1998”    sales_unit@icao.org

Knüpffer, 1997 “AUTOMATION OF AVIATION FORECASTS – THE PROJECTS AUTO_TAF AND AUTO GAFOR”    Seventh Conference on Aviation, Range and Aerospace Meteorology 77th AMS Annual Meeting, pp 444-449